| [Facility Name/Logo] | Care and Disinfection of noncritical patient care equipment |
| --- | --- |
|  | Policy Number: |

| Section: |  | Origin date: |  |
| --- | --- | --- | --- |
| Owner: |  | Effective Date: |  |
| Approved by: |  | Last Revised Date: |  |
| Prepared by: |  | Next Review Date: |  |

- Policy Description

This policy outlines a standardized approach for selecting and disinfecting patient care equipment to minimize the risk of pathogen transmission.

- Rationale

Proper cleaning and disinfection of patient care equipment are essential for preventing healthcare-associated infections (HAIs) and maintaining a safe care environment. Equipment used in patient care must be capable of being cleaned and disinfected by using facility-approved methods and products. The process for identifying reusable patient care equipment and their cleaning and disinfection processes begins during the product evaluation process prior to brining it into the facility.

- Definitions

(General policies statements are included here, facilities should modify to their facility’s needs.)

  - Low-Level Disinfection – A process that removes most vegetative bacteria, fungi, and some viruses but does not eliminate spores. Used for items that contact intact skin.

  - Intermediate-Level Disinfection – A process that eliminates mycobacteria, most viruses, and most fungi but does not necessarily kill bacterial spores.

  - Cleaning – The removal of visible soil, blood, protein substances, and other debris from surfaces and objects before disinfection.

  - Disinfection – A process that eliminates many or all pathogenic microorganisms, except bacterial spores, from inanimate objects.

  - Non-Critical Equipment – Devices that contact only intact skin and require low-level disinfection.

  - Spaulding Classification- A system used to determine the level of disinfection or sterilization required for medical devices and instruments based on their intended use.

| Category (Spaulding Class) | Definition | Examples | Regulated by | Minimum reprocessing requirements |
| --- | --- | --- | --- | --- |
| Non-critical | Objects that touch only intact skin | Blood pressure cuffs, stethoscopes, high-touch patient care items | EPA | Low-level disinfection |
| Semi-critical | Objects that touch mucous membranes or non-intact skin | Endoscopes, laryngoscopes, Respiratory Therapy equipment, Vaginal specula | EPA | High-level disinfection (HLD) |
| Critical | Objects which enter normally sterile tissue or the vascular system | Implants, surgical instruments, needles for IV | FDA | Sterilization |

- Policy Guidelines

  - Reprocessing of Non- Critical Patient Care Equipment

    - All reusable non-critical patient care equipment must be disinfected using a hospital-approved, EPA-registered disinfectant.

    - Disinfection must follow the Device Manufacturer’s Instructions for Use (MIFU):

    - Shared equipment must be cleaned and disinfected at point of use.

    - Terminal cleaning of non-critical equipment must be performed outside patient care zones, per facility-specific guidelines.

    - Cleaned and disinfected devices must be stored in a way that protects items from contamination, including:

      - Separation of clean and dirty equipment.

      - Clear labeling indicating that items are ready for use.

    - Regular maintenance and inspection of reusable devices must be conducted to ensure functionality and cleanability.

| Type of Equipment | Frequency | Method |
| --- | --- | --- |
| Shared equipment | Before and after each use | Clean and disinfect |
| Dedicated equipment in an isolation room | According to patient care area cleaning schedule | Method based on risk level of the patient care area |
| All equipment | After patient transfer or discharge (terminal cleaning) | Clean and disinfect |

  - Use of Disinfectants

    - Chemical MIFUs must be strictly followed

    - Develop Appendix for list of hospital approved disinfectants

    - All items requiring low-level disinfection must be exposed to the solution per manufacturer recommended dwell time.

    - Expiration dates, as well as mix and use dates (if applicable) must comply with the MIFU.

    - Safety Data Sheets (SDS) for hospital approved disinfectants may be found. Insert protocol for facility

  - Equipment Selection and Cleaning

    - All equipment purchased for use in this facility must be compatible with facility-approved disinfectants.

    - If an item cannot be properly disinfected, the Infection Prevention team must be consulted to explore alternative options.

    - If plastic coverings are recommended by the manufacturer, items must still be clean these items with the same frequency, inspect coverings for damage on a regular basis, and repair or replace them as needed.

    - Single use disposable patient care items may not be disinfected for reuse.

  - Cleaning Process

    - Visible blood, bioburden, and debris must be removed before disinfection can occur.

      - This may require a two-step cleaning process where the soil is removed to clean the device then a germicidal wipe is applied for the necessary dwell time in order to disinfect the device.

    - Follow the recommended dwell time for the manufacturer.

      - Opportunity to refer to your specific protocols here or to refer to an appendix.

  - Staff Responsibilities

    - Staff roles and responsibilities for cleaning and disinfection in the facility are outlined here or refer to an appendix.

| Area/ Item | Staff Responsible |  |  | Method (Chemical Used) |  |  | Frequency |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Patient Room | Nursing | EVS | Other: (e.g., admin assistant) | Quat | Bleach | Hydrogen Peroxide | Daily (Routine) | Point of Care/ As needed | Discharge/ Terminal |
| Bed Rails |  | X |  |  |  | X | X |  | X |
| Patient Bathroom |  | X |  |  |  |  | X | X | X |
| Patient Bed |  |  |  |  |  |  |  |  |  |
| Call Light |  |  |  |  |  |  |  |  |  |
| Add more as needed |  |  |  |  |  |  |  |  |  |
| Add more as needed |  |  |  |  |  |  |  |  |  |

- Education

  - Competency based training program upon hire, yearly and as needed

  - Who receives training: per your facility’s policy

  - What is included in training program per your facility’s policy

- Documentation

- Responsibilities: Infection Prevention and Environmental Services

- Responsible for content/Approval authority: Infection Prevention and Environmental Services

- References

- Related policies

- Approval Signatures

| Department Approval Dr. XXXXXX Title: Medical Director/Department Chair Department: | Administrative Approval XXXXXXXXX Title: Director/Manager etc. Department: |
| --- | --- |

- Appendix(es):